# Modular SSVEP BCI Controller

## System Architecture, Prototype 1 Specification, Interfaces, Development Stages, and Expansion Plan

Document purpose:
This document defines the technical architecture and development roadmap for the team's SSVEP-based Brain-Computer Interface (BCI) controller.

The primary purpose is to keep the entire team aligned on:

- what the system receives from EEG hardware,

- how that signal is represented internally,

- how the signal is processed,

- how SSVEP targets are detected/classified,

- how a classification becomes a trigger,

- how the trigger reaches the game/application,

- which parts are fixed architectural contracts,

- which parts are experimental,

- and how the system will scale from 1 channel / 4 frequencies to 8–16 channels / larger target sets.

# 1. Project Objective

The system is an EEG-based SSVEP controller.

The user observes visual targets that flicker at known frequencies.

The EEG acquisition hardware captures brain activity.

The software analyzes a continuously arriving EEG signal and determines whether the signal contains sufficient evidence for one of the configured SSVEP targets.

The final output of the BCI stack is a semantic command/event, not a direct game-specific implementation.

The fundamental pipeline is:

USER

│

│ looks at visual stimulus

▼

VISUAL STIMULUS

│

│ flickering at target frequency

▼

BRAIN / VISUAL CORTEX

│

│ SSVEP response

▼

EEG ELECTRODES

│

▼

EEG AMPLIFIER / ADC

│

▼

SENSOR EMBEDDED BOARD

│

│ digital EEG samples

▼

ACQUISITION ADAPTER

│

▼

STANDARDIZED EEG STREAM

│

▼

BUFFER

│

▼

PREPROCESSING

│

▼

WINDOWING

│

├───────────────┐

▼               ▼

CLASSICAL       NEURAL

DECODER         DECODER

│               │

└───────┬───────┘

▼

TARGET SCORES

│

▼

DECISION ENGINE

│

▼

TRIGGER / CLASSIFIED

FREQUENCY

│

▼

COMMAND MAPPING

│

▼

GAME

The architecture must preserve this separation.

# 2. Core Architectural Principle

The project must NOT be designed as:

1 EEG channel

+

4 hard-coded frequencies

+

one neural network

+

one game

Instead, it must be designed as:

N EEG channels

+

N configurable SSVEP targets

+

pluggable signal-processing algorithms

+

pluggable classifiers

+

independent decision logic

+

independent application/game interface

Prototype 1 will simply use:

N = 1 channel

N_targets = 4

The architecture must allow:

1 → 8 → 16 channels

and:

4 → N target frequencies

without rewriting the entire system.

# 3. What Is Fixed and What Is Experimental

This distinction is important.

## 3.1 Architectural contracts

These should remain stable:

- EEG input abstraction

- timestamped samples

- channel metadata

- sampling-rate metadata

- physical-unit representation

- ring buffer

- preprocessing interface

- windowing interface

- decoder interface

- confidence representation

- decision engine

- event/command interface

- game interface

## 3.2 Experimental components

These may change:

- filter parameters

- window duration

- window step

- FFT/PSD implementation

- harmonic selection

- SNR calculation

- model architecture

- number of neural-network filters

- training strategy

- confidence threshold

- temporal voting rules

- target frequencies

- number of channels

- channel selection

- reference/montage

- game mappings

This prevents experiments from turning into architectural rewrites.

# 4. System Boundary

The BCI software begins at the point where the sensor embedded board provides digitized EEG samples.

Everything before this boundary belongs to the hardware/acquisition subsystem.

Everything after this boundary belongs to the BCI software stack.

┌──────────────────────── HARDWARE DOMAIN ────────────────────────┐

Electrodes

↓

Analog front end

↓

Amplifier

↓

ADC

↓

Embedded processor

↓

USB / UART / BLE / Ethernet / vendor SDK

└───────────────────────────┬────────────────────────────────────┘

│

SYSTEM INTERFACE

│

┌───────────────────────────▼────────────────────────────────────┐

Acquisition Adapter

↓

Signal Validation

↓

Timestamping

↓

Ring Buffer

↓

Preprocessing

↓

Windowing

↓

Decoder

↓

Decision Engine

↓

Event API

└────────────────────────────────────────────────────────────────┘

This boundary is one of the most important architectural decisions in the project.

The machine-learning code must not know whether the EEG came from:

- UART,

- USB,

- BLE,

- TCP,

- LSL,

- a vendor SDK,

- a Raspberry Pi,

- an embedded Linux board,

- or another acquisition computer.

It receives a standardized EEG stream.

# 5. EEG Input Contract

## 5.1 General requirement

The software shall accept EEG as a continuous sequence of samples:

channels × time

Internally, the preferred representation is:

samples[channel][time]

or equivalently:

shape = (channels, samples)

For machine-learning batches:

shape = (batch, channels, samples)

This is important because Prototype 1 will have:

channels = 1

but future versions will have:

channels = 8

channels = 16

The software must therefore never assume that the EEG has only one channel.

# 6. Prototype 1 Hardware Input Profile

Prototype 1 should establish a clearly defined baseline.

## P1 EEG profile

| Parameter | Prototype 1 |
| --- | --- |
| EEG channels | 1 |
| Sampling rate | 256 Hz |
| Target frequencies | 8.57, 10, 12, 15 Hz |
| Expected target count | 4 |
| Signal type | Differential EEG |
| Primary signal region | Visual/occipital response |
| Reference dataset | Acampora single-channel SSVEP dataset |
| Prototype acquisition | Real sensor board when available |
| Internal signal unit | µV |
| Processing representation | floating-point |
| Real-time operation | Yes, after offline validation |
| Neural model | EEGNet-inspired SSVEP model |
| Classical baseline | Required |
| Decision output | target / NONE / confidence |
| Game output | semantic event |

The existing reference dataset was recorded at 256 Hz with four target frequencies and 16-second trials. It used a single differential EEG channel with Fpz/Oz electrodes and an additional reference electrode.

The reference dataset uses 10-bit ADC values from 0–1023, but the future software architecture must not be locked to a 10-bit ADC.

That ADC resolution belongs to the particular reference acquisition system, not to the BCI architecture.

# 7. Recommended Hardware-to-Software Signal Standard

The acquisition hardware may internally use any ADC resolution.

The software should normalize the incoming data to a common representation.

## Required normalized signal

Each EEG sample should eventually become:

timestamp

channel_id

value_uV

For multiple channels:

timestamp

channel_1_uV

channel_2_uV

...

channel_N_uV

The preferred internal representation is:

float32 or float64

with values expressed in:

µV

This means the neural network and signal-processing modules do not need to know whether the hardware ADC was:

10-bit

12-bit

16-bit

24-bit

# 8. Mandatory Acquisition Metadata

Every EEG stream must provide, either at startup or through configuration:

sampling_rate_hz

channel_count

channel_names

channel_units

electrode locations

reference

montage / derivation

ADC information

gain / calibration

physical range

timestamp information

device identifier

firmware version

For example:

{

"sampling_rate_hz": 256,

"channel_count": 1,

"channels": [

{

"id": 0,

"name": "EEG1",

"electrode": "Oz",

"reference": "Fpz"

}

],

"unit": "uV",

"device": "prototype_eeg_board",

"version": "0.1"

}

The exact JSON representation is not mandatory.

The information itself is mandatory.

# 9. Why Calibration Metadata Matters

Raw ADC values must never be interpreted as EEG amplitude without knowing how to convert them.

For example:

ADC count

↓

offset correction

↓

gain calibration

↓

physical EEG value

↓

µV

The conversion must be explicitly defined by the acquisition adapter.

For the current reference dataset, the ADC values are 10-bit and the paper describes a ±0.39 mV dynamic range. Our existing software converts the ADC counts into an approximate µV representation based on that calibration.

That conversion must NOT simply be copied to future hardware.

Every new acquisition board must provide its own calibration information.

# 10. Electrode and Reference Information

The acquisition subsystem must explicitly identify:

active electrode(s)

reference electrode

ground / bias / DRL electrode

signal polarity

montage

For example:

EEG channel:

Oz - Fpz

Reference:

earlobe / designated reference

Bias / DRL:

hardware-dependent

The reference dataset used Fpz and Oz as the active differential electrodes and an earlobe/DRL reference.

Future 8/16-channel systems should use explicit electrode labels rather than:

CH1

CH2

CH3

...

where possible.

Standard EEG naming such as:

Fpz

Oz

O1

O2

Fz

Cz

Pz

...

makes later montage handling and re-referencing much easier.

EDF+ also provides standardized EEG labels and electrode-position conventions, which is useful when designing our metadata model even if EDF+ is not used for live transport.

# 11. Live Streaming Interface

The BCI software should not communicate directly with hardware-specific packet formats throughout the application.

Instead:

Hardware Driver

↓

Acquisition Adapter

↓

Internal EEG Stream

The acquisition adapter is the only hardware-dependent component.

For example:

UART EEG board

↓

UART Adapter

↓

Normalized EEG Stream

or:

USB EEG board

↓

USB Adapter

↓

Normalized EEG Stream

or:

Vendor SDK

↓

SDK Adapter

↓

Normalized EEG Stream

# 12. Optional LSL Integration

Lab Streaming Layer (LSL) is a good candidate for the development/integration layer.

It is designed around exactly this kind of architecture:

device/application

↓

LSL stream

↓

processing application

LSL also supports metadata describing the stream.

Therefore, one possible development architecture is:

EEG Board

↓

Board Driver

↓

Acquisition Adapter

↓

LSL EEG Stream

↓

BCI Processing

However:

LSL should not be treated as a mandatory hardware requirement.

If the embedded board uses a more appropriate transport, the acquisition adapter can translate it into the same internal representation.

LSL is therefore an interoperability option, not the core architecture.

# 13. EDF/EDF+ Usage

EDF/EDF+ should primarily be considered a:

recording

storage

exchange

offline analysis

format.

It should not become the real-time internal representation.

EDF defines signal metadata such as physical/digital ranges, sampling information, labels, units, and prefiltering information, making it useful as an archival/reference format.

Recommended architecture:

LIVE:

Hardware → Stream → BCI

RECORDING:

Hardware → Stream → Recorder → EDF+/other dataset format

not:

Hardware → constantly write EDF → read EDF → process

# 14. Acquisition Adapter Responsibilities

The acquisition adapter is responsible for:

- Connecting to the device.

- Reading packets.

- Validating packets.

- Detecting packet loss.

- Converting raw ADC values.

- Applying calibration.

- Assigning timestamps.

- Reconstructing channel ordering.

- Detecting disconnects.

- Exposing normalized samples.

It must NOT:

- classify SSVEP,

- run the neural network,

- decide game commands,

- contain game logic.

# 15. Acquisition Validation

Before a sample enters the BCI pipeline, the system should verify:

Is the packet valid?

Is the sample count correct?

Are timestamps valid?

Are channels present?

Is the sample within expected range?

Has data been dropped?

Has the device disconnected?

Example status:

DEVICE_CONNECTED

STREAMING

PACKET_LOSS

TIMESTAMP_ERROR

SIGNAL_OUT_OF_RANGE

DEVICE_DISCONNECTED

These states should be visible to the application.

# 16. Sampling Rate

Prototype 1:

Fs = 256 Hz

Therefore:

1 second = 256 samples

0.5 second = 128 samples

0.75 second = 192 samples

1 second = 256 samples

1.5 seconds = 384 samples

2 seconds = 512 samples

4 seconds = 1024 samples

The processing system should derive sample counts from:

samples = round(window_seconds * sampling_rate)

rather than hard-coding 256 samples everywhere.

This is important when future hardware changes sampling rate.

# 17. Ring Buffer

The system should continuously maintain a rolling buffer.

Example:

Incoming EEG

↓

┌───────────────────────────────┐

│        RING BUFFER             │

│                               │

│ old ←───────────────→ newest │

└───────────────────────────────┘

The buffer should contain enough samples for:

maximum model window

+

processing margin

For Prototype 1:

recommended capacity:

at least 4–8 seconds

The ring buffer must be independent of the classifier.

# 18. Windowing

The classifier does not need to process the entire EEG history.

It receives a rolling window:

continuous EEG

──────────────────────────────────────>

┌─────────────┐

│   WINDOW    │

└─────────────┘

↓

decode

Prototype 1 should initially evaluate:

0.75 s

1.0 s

1.5 s

2.0 s

rather than assuming that one duration is optimal.

For the first reliable end-to-end implementation, use:

default window = 2.0 s

step = 250 ms

This prioritizes reliable detection over minimum latency.

After the complete system works, shorter windows can be benchmarked.

# 19. Why We Do Not Optimize Latency First

The system has two different latencies:

algorithmic/information latency

and:

computation latency

A neural network may execute in:

5 ms

but that does not mean we can reliably identify a stimulus from only 5 ms of EEG.

The limiting factor is often the amount of EEG evidence required.

Therefore:

First:

Can we detect correctly?

Then:

How short can the observation window become?

Then:

How fast can the implementation execute?

# 20. Preprocessing Pipeline

Prototype 1 preprocessing:

raw EEG

↓

ADC calibration

↓

physical units (µV)

↓

DC / baseline removal

↓

band-pass filtering

↓

optional artifact handling

↓

normalized model input

The preprocessing implementation must be configurable.

Example configuration:

sampling_rate = 256 Hz

high_pass = configurable

low_pass  = configurable

notch = configurable

normalization = configurable

# 21. Prototype 1 Frequency Region

Our initial SSVEP targets are:

8.57 Hz

10 Hz

12 Hz

15 Hz

The existing analysis pipeline also examines the second harmonics:

17.14 Hz

20 Hz

24 Hz

30 Hz

These harmonics are useful because SSVEP responses are not necessarily confined to the fundamental frequency.

Prototype 1 therefore keeps harmonic information available to the classical decoder and potentially to future feature engineering.

# 22. Classical Decoder

A classical decoder must exist before the neural network becomes the sole decision mechanism.

This gives us a baseline.

Prototype 1 classical processing:

EEG window

↓

preprocessing

↓

PSD / spectral analysis

↓

target frequency power

↓

harmonic power

↓

local noise estimation

↓

SNR

↓

target scores

For each target:

F1 score

F2 score

F3 score

F4 score

The classical system should also provide:

best target

confidence

margin

individual scores

# 23. Why the Classical Decoder Is Mandatory

It serves four purposes.

### 1. Debugging

If the neural network fails, we still know whether the EEG itself contains recognizable SSVEP structure.

### 2. Scientific baseline

We can compare:

PSD/SNR

vs

Neural Network

### 3. Development safety

The team can continue working on acquisition and UI without waiting for ML training.

### 4. Hybrid architecture

Eventually:

Classical score

+

Neural score

↓

Fusion

↓

Final decoder

may be useful.

# 24. Neural Decoder

The neural system should be treated as a decoder, not as the entire BCI system.

Conceptually:

EEG window

↓

SSVEPNet / EEGNet-inspired network

↓

class probabilities

Example:

F1 = 0.04

F2 = 0.08

F3 = 0.82

F4 = 0.03

NONE = 0.03

The neural network should not directly execute:

JUMP()

or:

MOVE_LEFT()

It should only classify EEG.

# 25. Neural Network Architecture Strategy

The network should be inspired by EEGNet but adapted to our problem.

The original EEGNet separates temporal filtering from spatial filtering and uses depthwise/separable convolutions to keep the model compact. The paper also emphasizes that the temporal convolution can behave like learned narrow-band frequency filters.

Prototype 1:

Input

(batch, 1, samples)

↓

Temporal convolution

↓

Normalization

↓

Nonlinearity

↓

Temporal feature extraction

↓

Compact classifier

↓

F1/F2/F3/F4/(NONE)

We should NOT artificially pretend that one channel contains useful spatial information.

# 26. Future Multi-Channel Neural Architecture

When we reach 8/16 channels:

Input

(batch, channels, samples)

↓

Temporal filtering

↓

Spatial/depthwise filtering

↓

Feature extraction

↓

Separable convolution

↓

Classifier

The architecture then becomes much closer to the original EEGNet design.

This is why the input tensor must already be defined as:

(batch, channels, time)

from Prototype 1.

# 27. Important EEGNet Parameter Rule

Do not blindly copy kernel sizes from the EEGNet paper.

The paper's architecture used a 128 Hz sampling rate and a 64-sample temporal kernel, corresponding to approximately 500 ms of signal.

At:

128 Hz:

64 samples ≈ 500 ms

but at:

256 Hz:

64 samples ≈ 250 ms

Therefore, architecture parameters must be translated into time, not merely copied as sample counts.

At 256 Hz:

250 ms ≈ 64 samples

500 ms ≈ 128 samples

The temporal receptive field must therefore be selected based on the intended frequency response and SSVEP task.

# 28. Number of Classes

Prototype 1:

F1

F2

F3

F4

The architecture should nevertheless support:

N targets

rather than hard-coding four outputs.

Recommended future class structure:

NONE

F1

F2

F3

...

FN

The NONE state is important.

The controller must be allowed to say:

"No command."

rather than forcing every EEG window into one of the available commands.

# 29. Decoder Output Contract

Every decoder should return a common structure.

Conceptually:

{

"target": "F3",

"confidence": 0.87,

"scores": {

"NONE": 0.05,

"F1": 0.04,

"F2": 0.08,

"F3": 0.87,

"F4": 0.03

},

"timestamp": ...,

"window_start": ...,

"window_end": ...

}

A classical decoder should return the same conceptual structure.

A neural decoder should return the same conceptual structure.

This lets the decision layer remain unchanged.

# 30. Decision Engine

This is one of the most important modules.

The classifier says:

"What does this EEG window look like?"

The decision engine says:

"Is this strong and stable enough to become a command?"

Pipeline:

Decoder output

↓

confidence check

↓

margin check

↓

temporal stability

↓

NONE check

↓

debounce

↓

refractory period

↓

TRIGGER

# 31. Temporal Stability

One classification should not automatically become a command.

Example:

window 1 → F3

window 2 → F3

window 3 → F3

window 4 → F3

is much more convincing than:

window 1 → F1

window 2 → F4

window 3 → F3

window 4 → F2

The decision engine should therefore maintain a short history.

Possible mechanisms:

majority voting

weighted voting

exponential smoothing

probability accumulation

confidence hysteresis

The exact method is an experimental parameter.

# 32. Refractory Period

After a successful trigger:

F3 → JUMP

the system should temporarily prevent repeated triggering of the same command.

Otherwise one sustained gaze may produce:

JUMP

JUMP

JUMP

JUMP

JUMP

instead of:

JUMP

Prototype 1 should therefore have a configurable:

refractory period

or:

command cooldown

# 33. Command Mapping Layer

This layer converts EEG targets into application-independent semantic events.

Example:

F1 → LEFT

F2 → RIGHT

F3 → JUMP

F4 → SELECT

But the decoder itself does not know this mapping.

Configuration:

target frequency → semantic action

This allows the same BCI decoder to control:

- games,

- robot interfaces,

- menus,

- wheelchair interfaces,

- media controls,

- experimental applications.

# 34. Game Interface

The game should receive events like:

BCIEvent {

action: LEFT

confidence: 0.91

timestamp: ...

}

The game should NOT receive:

raw EEG

FFT arrays

neural network tensors

PSD data

model internals

unless it explicitly needs them for visualization.

This separation allows the BCI system to be developed independently from the game.

# 35. Recommended Software Module Structure

The project should approximately follow:

bci/

│

├── acquisition/

│   ├── base.py

│   ├── device_adapter.py

│   ├── packet_parser.py

│   └── validation.py

│

├── stream/

│   ├── eeg_stream.py

│   ├── timestamp.py

│   └── ring_buffer.py

│

├── preprocessing/

│   ├── filters.py

│   ├── normalization.py

│   └── artifacts.py

│

├── windowing/

│   └── window_manager.py

│

├── features/

│   ├── psd.py

│   ├── snr.py

│   ├── harmonics.py

│   └── references.py

│

├── decoders/

│   ├── base.py

│   ├── classical_ssvep.py

│   ├── ssvepnet.py

│   └── fusion.py

│

├── decision/

│   ├── confidence.py

│   ├── temporal.py

│   ├── debounce.py

│   └── trigger.py

│

├── events/

│   └── bci_event.py

│

├── game/

│   └── interface.py

│

├── config/

│   └── prototype1.yaml

│

├── recording/

│   └── recorder.py

│

├── evaluation/

│   ├── metrics.py

│   └── reports.py

│

└── tools/

├── visualizer.py

└── diagnostics.py

The exact language/package structure can change.

The conceptual boundaries should not.

# 36. Prototype 1 Configuration

Prototype 1 should have one explicit configuration.

Conceptually:

sampling_rate_hz: 256

channels:

- id: 0

name: EEG1

targets:

- id: F1

frequency_hz: 8.57

- id: F2

frequency_hz: 10.0

- id: F3

frequency_hz: 12.0

- id: F4

frequency_hz: 15.0

window:

duration_seconds: 2.0

step_seconds: 0.25

preprocessing:

high_pass_hz: configurable

low_pass_hz: configurable

notch_hz: configurable

decoder:

classical: enabled

neural: enabled

decision:

confidence_threshold: experimental

stability_requirement: experimental

refractory_seconds: experimental

The actual thresholds must be determined experimentally.

They should never be treated as scientifically established merely because they worked on one recording.

# 37. Prototype 1 Development Sequence

Prototype 1 should NOT be developed as one giant system.

It should be developed in controlled stages.

## P1-A — Signal acquisition

Goal:

Hardware → software

Verify:

- device connects,

- samples arrive,

- sample rate is correct,

- channel count is correct,

- no unexpected packet loss,

- timestamps behave correctly.

No ML.

No game.

## P1-B — Signal visualization

Build:

live EEG waveform

Display:

amplitude

sample rate

channel

signal range

packet loss

connection state

This stage proves that we are receiving a plausible signal.

## P1-C — Recording

Record the live stream.

Every recording must contain:

EEG samples

timestamps

channel metadata

sampling rate

device information

experiment markers

target frequency

This creates our own training/evaluation dataset.

# 38. P1-D — Offline Classical Decoder

Before real-time ML:

recorded EEG

↓

preprocessing

↓

windowing

↓

PSD/SNR decoder

↓

classification

Evaluate:

- accuracy,

- confusion matrix,

- confidence,

- window duration,

- false detection,

- missed detection.

# 39. P1-E — Neural Model

Train the first SSVEPNet/EEGNet-inspired model.

Important:

The neural network must be compared against the classical baseline.

The question is not:

"Can CNN classify EEG?"

The real question is:

"Does the neural model provide useful improvement

for this SSVEP controller at the same observation window?"

# 40. P1-F — Real-Time Neural Inference

Once offline inference is working:

live EEG

↓

ring buffer

↓

window

↓

preprocessing

↓

neural inference

↓

prediction

Measure:

inference time

CPU usage

memory

prediction stability

Do not optimize hardware acceleration before correctness is established.

# 41. P1-G — Decision Engine

Connect:

prediction

↓

confidence

↓

temporal stability

↓

trigger

At this stage, do not connect to a complicated game.

Use a simple diagnostic output:

[14:32:10.512]

PREDICTION = F3

CONFIDENCE = 0.89

STATUS = STABLE

TRIGGER = YES

# 42. P1-H — Game Integration

Only after the complete BCI pipeline works:

EEG

↓

decoder

↓

decision

↓

BCI event

↓

game

The game should be the last component connected.

This prevents game bugs from being mistaken for EEG/ML bugs.

# 43. Prototype 1 Acceptance Criteria

Prototype 1 should not be considered complete merely because:

the neural network runs

or:

the graph looks correct

It is complete when the following pipeline works:

EEG hardware

↓

live acquisition

↓

validated samples

↓

buffer

↓

preprocessing

↓

window

↓

decoder

↓

decision

↓

trigger

↓

game action

and the system can repeat this reliably.

# 44. Prototype 1 Metrics

The team should record at least:

### Classification

- accuracy

- balanced accuracy

- macro F1

- confusion matrix

- per-target accuracy

### Controller behavior

- false-trigger rate

- missed-trigger rate

- correct-trigger rate

- detection latency

- time-to-first-correct-detection

- repeated-trigger rate

- abstention/NONE rate

### Signal quality

- amplitude range

- noise level

- target SNR

- packet loss

- dropped samples

- electrode/contact quality if available

### Runtime

- preprocessing latency

- model inference latency

- decision latency

- total software latency

- CPU usage

- memory usage

# 45. Dataset Split Rules

This is critical.

The current dataset contains 11 subjects, four target conditions, and 16-second recordings. It is useful for initial algorithm development, but it is not a continuous game-control dataset.

Do NOT randomly split overlapping windows from the same recording into training and testing.

That would allow extremely similar pieces of the same EEG recording to appear on both sides of the split.

For initial within-subject experiments:

recording blocks

should be separated.

For cross-subject evaluation:

train subjects

↓

test subject

is preferable.

A leave-one-subject-out strategy is especially useful later.

# 46. Important Limitation of the Existing Dataset

The reference CSV format has:

F1 column

F2 column

F3 column

F4 column

where each column corresponds to a separate known stimulus recording.

It is NOT equivalent to:

continuous EEG

F1

F1

F1

NONE

F3

F3

NONE

F2

...

Therefore it can help us develop:

classification

feature extraction

model architecture

but it cannot fully validate:

real-time transition latency

false triggering during idle periods

target switching behavior

continuous game control

Those require our own continuous recordings.

# 47. Required Prototype 1 Recording Protocol

Once real hardware is available, the team should create recordings specifically for the controller.

A useful initial protocol should contain:

REST / NONE

↓

F1

↓

REST

↓

F2

↓

REST

↓

F3

↓

REST

↓

F4

and eventually randomized target sequences.

Every stimulus transition must generate an event marker.

Example:

timestamp       event

--------------------------------

10.000          REST

15.000          F1_START

20.000          REST

25.000          F3_START

30.000          REST

35.000          F2_START

This is extremely important for measuring actual detection latency.

# 48. Stimulus System Must Also Be Modular

The stimulus generator should have its own configuration:

target ID

frequency

visual appearance

position

duration

refresh rate

action mapping

The BCI decoder should not hard-code the screen layout.

Example:

F1 = 8.57 Hz → left

F2 = 10 Hz   → right

F3 = 12 Hz   → jump

F4 = 15 Hz   → select

This is configuration, not architecture.

# 49. Increasing the Number of Frequencies

When moving from four targets to more targets:

4

↓

6

↓

8

↓

12

...

the following should NOT need redesign:

- acquisition

- buffer

- preprocessing

- event interface

- game interface

- logging

- recording

Only the following should change:

target configuration

decoder output dimension

stimulus configuration

training dataset

decision thresholds

Potentially:

model architecture

if the target set becomes sufficiently large.

# 50. Frequency Planning

Adding frequencies is not simply:

"add more numbers to the list."

The team must consider:

- distance between target frequencies,

- monitor refresh rate,

- harmonics,

- frequency resolution,

- observation window,

- visual comfort,

- SSVEP response strength,

- classifier confusion.

For example:

8.57 Hz

10 Hz

are much closer than:

10 Hz

15 Hz

Therefore the required observation time may increase as targets become more densely packed.

The target-frequency planner should eventually be a separate module.

# 51. Scaling From 1 to 8 Channels

The software architecture should already support:

channels = 8

without changing the acquisition API.

The changes occur mainly in:

hardware adapter

channel metadata

preprocessing

model input

spatial filtering

The tensor changes from:

(batch, 1, samples)

to:

(batch, 8, samples)

The decoder interface remains unchanged.

# 52. Scaling From 8 to 16 Channels

The same principle applies.

(batch, 16, samples)

The model can now learn spatial patterns.

Potential additions:

common average reference

channel selection

spatial filters

depthwise spatial convolution

channel dropout

artifact/channel-quality detection

These should be introduced only when supported by real data.

# 53. Channel Quality Module

Once multiple channels exist, the system should eventually determine whether a channel is healthy.

Possible measurements:

flatline detection

saturation

variance

high-frequency noise

line noise

electrode impedance if hardware provides it

correlation with neighboring channels

artifact probability

A bad channel should be able to produce:

CHANNEL_DEGRADED

rather than silently poisoning the classifier.

# 54. Reference and Re-Referencing

With multiple channels, the architecture should support different montages.

For example:

raw channels

↓

reference selection

↓

montage

↓

model

Possible approaches may include:

common reference

linked reference

average reference

bipolar derivations

The correct choice should be determined experimentally.

The important architectural requirement is that reference handling belongs to preprocessing/montage, not inside the game layer.

# 55. Multi-Channel EEGNet Evolution

The expected progression is:

## Prototype 1

1 channel

↓

temporal filtering

↓

compact classifier

## Prototype 2

8 channels

↓

temporal filtering

↓

spatial/depthwise filtering

↓

separable feature extraction

↓

classifier

## Prototype 3

16 channels

↓

temporal + spatial filtering

↓

SSVEP-specific feature representation

↓

classifier

This is much cleaner than developing an unrelated model for every hardware revision.

# 56. Classical + Neural Hybrid Architecture

Eventually we should be able to produce:

EEG

│

┌─────────┴─────────┐

│                   │

▼                   ▼

Classical Decoder      Neural Decoder

│                   │

▼                   ▼

scores              scores

│                   │

└─────────┬─────────┘

▼

Fusion

│

▼

Decision Engine

For example:

classical confidence = 0.81

neural confidence    = 0.88

agreement             = YES

This could become stronger evidence than either system alone.

But this is a future optimization.

Prototype 1 must first establish each component independently.

# 57. Logging

Every real-time inference should optionally be loggable.

Example:

timestamp

window_start

window_end

target

confidence

all_class_scores

classical_scores

neural_scores

decision

trigger

channel_quality

processing_latency

This allows us to answer:

Why did the system trigger here?

without guessing.

# 58. Debug Mode

The system should support a debug mode that exposes:

raw EEG

filtered EEG

PSD

target frequencies

harmonics

decoder scores

model prediction

confidence

decision state

trigger state

This is extremely useful during hardware development.

A production mode can hide most of these details.

# 59. Visualization

The diagnostic UI should ideally show:

┌─────────────────────────────────────────────┐

│ EEG waveform                                │

├─────────────────────────────────────────────┤

│ PSD                                         │

│                                             │

│  8.57   10   12   15 Hz                    │

├─────────────────────────────────────────────┤

│ F1 █████                                    │

│ F2 ███                                      │

│ F3 █████████████                            │

│ F4 ██                                       │

├─────────────────────────────────────────────┤

│ Prediction: F3                              │

│ Confidence: 0.87                            │

│ Decision: STABLE                            │

│ Trigger: YES                                │

└─────────────────────────────────────────────┘

This is a development tool, not part of the core decoder.

# 60. Error Handling

The system must distinguish:

NO EEG DATA

SIGNAL BAD

SIGNAL TOO NOISY

NO TARGET DETECTED

TARGET DETECTED

DEVICE DISCONNECTED

DECODER ERROR

MODEL ERROR

These are not the same state.

Especially:

NO TARGET

must not be treated as:

F1

# 61. State Machine

The complete controller should eventually behave like:

┌──────────────┐

│ DISCONNECTED │

└──────┬───────┘

│

▼

┌──────────────┐

│ CONNECTING   │

└──────┬───────┘

│

▼

┌──────────────┐

│ STREAMING    │

└──────┬───────┘

│

▼

┌──────────────┐

│ BUFFERING    │

└──────┬───────┘

│

▼

┌──────────────┐

│ CLASSIFYING  │

└──────┬───────┘

│

┌──────┴───────┐

▼              ▼

NONE          CANDIDATE

│

▼

STABILIZING

│

▼

TRIGGER

│

▼

REFRACTORY

│

└──────→ CLASSIFYING

This state machine should be separate from the neural network.

# 62. Security of the Architecture Against Future Changes

Future changes should preferably affect only one layer.

Examples:

### Change EEG hardware

New hardware

↓

new acquisition adapter

Everything else remains unchanged.

### Change from 1 to 16 channels

channel configuration

+

preprocessing

+

model configuration

The event API remains unchanged.

### Change classifier

PSD → EEGNet

The decision engine remains unchanged.

### Change game

Game A → Game B

The EEG stack remains unchanged.

### Add frequencies

4 targets → 8 targets

The acquisition stack remains unchanged.

This is the definition of modularity we want.

# 63. Prototype Roadmap

## Prototype 1 — Single Channel / Four Targets

1 channel

4 frequencies

256 Hz

classical decoder

neural decoder

real-time stream

decision engine

simple game

Primary goal:

prove the complete end-to-end loop.

## Prototype 1.1 — Latency Optimization

Keep hardware unchanged.

Experiment:

2.0 s

1.5 s

1.0 s

0.75 s

Measure:

accuracy

false triggers

detection latency

Do not select the shortest window merely because it is faster.

Select the shortest window that satisfies the required reliability.

## Prototype 1.2 — Continuous Operation

Add:

NONE

rest periods

target transitions

randomized sequences

Measure real controller behavior.

# 64. Prototype 2 — 8 Channels

Add:

8-channel acquisition

channel metadata

multi-channel preprocessing

spatial filtering

multi-channel neural input

Keep:

decoder API

decision API

game API

logging

recording

unchanged.

# 65. Prototype 3 — 16 Channels

Add:

16-channel acquisition

channel quality

advanced referencing

spatial learning

artifact handling

The same event system remains.

# 66. Prototype 4 — Expanded Target Set

Add:

N frequencies

N classes

NONE

The target configuration becomes data-driven.

No frequency should be hard-coded into the game.

# 67. Prototype 5 — Optimized BCI Controller

Potential additions:

hybrid classical + neural decoding

adaptive thresholds

subject calibration

channel-quality weighting

personalized models

cross-subject models

model quantization

hardware acceleration

These are optimization stages, not Prototype 1 requirements.

# 68. What We Must NOT Do During Prototype 1

The team should explicitly avoid:

### Do not build a giant deep-learning architecture.

We have limited initial data.

### Do not immediately move to Transformers/LSTMs.

A compact temporal model is sufficient for the first experiment.

### Do not remove the classical baseline.

We need something interpretable to compare against.

### Do not optimize GPU acceleration first.

Correctness comes first.

### Do not build the game before the decoder is verified.

Otherwise debugging becomes ambiguous.

### Do not randomly split overlapping windows.

This can produce misleadingly high performance.

### Do not hard-code 1 channel throughout the code.

The model input must already support:

channels = N

### Do not hard-code four frequencies into the architecture.

Targets belong in configuration.

### Do not let the neural network directly issue game commands.

Classification and control must remain separate.

# 69. Team Ownership Boundaries

A useful team division is:

## Hardware team

Responsible for:

electrodes

AFE

ADC

sampling

embedded firmware

packet format

timestamps

device communication

signal calibration

## Acquisition/software team

Responsible for:

device driver

packet parser

stream interface

buffer

recording

metadata

signal validation

## Signal-processing team

Responsible for:

filters

PSD

SNR

harmonics

windowing

signal-quality analysis

## ML team

Responsible for:

dataset

training

EEGNet/SSVEPNet

evaluation

model export

inference

## BCI decision team

Responsible for:

confidence

temporal voting

debounce

refractory period

trigger rules

## Application/game team

Responsible for:

visual stimulus

game

command mapping

UI

The interfaces between these teams are more important than the internal implementation of each team.

# 70. Definition of Done for Each Module

Every module must have a test.

## Acquisition

Can we receive known samples correctly?

## Calibration

Can we convert raw values into physical units?

## Buffer

Does it preserve sample ordering and timestamps?

## Filter

Does it preserve the required SSVEP frequencies?

## Windowing

Does a 1-second window always contain the correct number of samples?

## Classical decoder

Can it identify known test signals?

## Neural decoder

Can it classify held-out recordings without leakage?

## Decision engine

Does it avoid triggering on unstable predictions?

## Event system

Does one trigger produce exactly one event?

## Game

Does each semantic event produce the expected action?

# 71. Synthetic Signal Tests

Before blaming EEG hardware, every decoder should be tested with synthetic signals.

For example:

x(t) = A sin(2π · 10t) + noise

and:

x(t) = A sin(2π · 12t)

+ B sin(2π · 24t)

+ noise

The decoder should identify the intended frequency.

This gives us a controlled test.

Pipeline:

synthetic EEG

↓

preprocessing

↓

decoder

If this fails, hardware is not the problem.

# 72. Hardware Test → Software Test → Model Test

Every failure should be isolated in this order:

1. Is the hardware producing valid data?

↓

2. Is the software receiving it correctly?

↓

3. Is preprocessing correct?

↓

4. Does classical SSVEP detection work?

↓

5. Does the neural model work?

↓

6. Does the decision engine work?

↓

7. Does the game respond?

Do not jump directly from:

game doesn't respond

to:

neural network must be wrong.

# 73. Reference Dataset Role

The current public/reference dataset should be treated as:

Algorithm Development Dataset

not:

Final Hardware Validation Dataset

It is valuable for:

- initial preprocessing,

- PSD analysis,

- feature extraction,

- model prototyping,

- classification experiments,

- architecture debugging.

But real hardware validation requires data collected from the team's actual acquisition system.

# 74. Required Team Vocabulary

Everyone on the team should use these terms consistently.

### Sample

One digitized EEG measurement.

### Channel

One EEG signal/derivation.

### Window

A finite sequence of consecutive EEG samples used for analysis.

### Target

A configured SSVEP stimulus frequency.

### Class

The decoder's classification category.

### Prediction

The decoder's current output.

### Candidate

A prediction that has not yet passed the decision rules.

### Trigger

A confirmed BCI event.

### Command

The semantic action generated by a trigger.

### Game action

The final application-specific behavior.

This distinction prevents:

prediction = trigger = action

from becoming an architectural mistake.

# 75. Canonical End-to-End Example

Suppose the user looks at:

12 Hz target

The complete system should behave like:

VISUAL TARGET

│

│ 12 Hz

▼

USER

│

▼

SSVEP RESPONSE

│

▼

EEG ELECTRODE

│

▼

ADC

│

▼

EMBEDDED BOARD

│

│ samples

▼

ACQUISITION ADAPTER

│

│ µV + timestamp

▼

RING BUFFER

│

▼

2-second WINDOW

│

▼

PREPROCESSING

│

▼

DECODER

│

├── F1 = 0.04

├── F2 = 0.08

├── F3 = 0.82

└── F4 = 0.06

│

▼

DECISION ENGINE

│

│ stable + confident

▼

TRIGGER

│

│ target = F3

▼

COMMAND MAPPER

│

│ F3 → JUMP

▼

BCI EVENT

│

▼

GAME

│

▼

JUMP

This is the fundamental product pipeline.

# 76. Final Architectural Contract

The team should treat the following as the core contract:

EEG HARDWARE

│

▼

HARDWARE ADAPTER

│

▼

NORMALIZED STREAM

│

▼

BUFFER

│

▼

PREPROCESSING

│

▼

WINDOWING

│

▼

┌───────────┴───────────┐

│                       │

▼                       ▼

CLASSICAL DECODER       NEURAL DECODER

│                       │

└───────────┬───────────┘

▼

COMMON OUTPUT

│

▼

DECISION ENGINE

│

▼

TRIGGER

│

▼

COMMAND MAPPER

│

▼

BCI EVENT

│

▼

GAME

The following must remain independent:

hardware

signal processing

decoder

decision logic

game

# 77. The Most Important Rule for the Project

When a new requirement appears, the team should ask:

"Which module should change?"

rather than:

"How do we modify the whole system?"

Examples:

New EEG board?

→ Acquisition adapter.

More channels?

→ Acquisition + preprocessing + model configuration.

More frequencies?

→ Target configuration + decoder/model.

Better classifier?

→ Decoder module.

Different trigger logic?

→ Decision engine.

Different game?

→ Command/game layer.

Lower latency?

→ Windowing + decoder + decision optimization.

Different electrode montage?

→ Acquisition metadata + preprocessing.

If a small requirement forces changes across the entire system, that is a signal that the architecture needs correction.

# 78. Prototype 1 Master Checklist

Before declaring Prototype 1 complete:

### Hardware

- EEG electrodes working

- EEG amplifier working

- ADC working

- sampling rate verified

- channel count verified

- calibration verified

- reference verified

- packet format documented

### Acquisition

- device adapter implemented

- timestamps implemented

- packet-loss detection implemented

- normalized µV stream implemented

- metadata implemented

### Signal pipeline

- ring buffer

- preprocessing

- windowing

- live visualization

- recording

### Classical decoder

- PSD

- target power

- harmonics

- SNR

- target scores

- confidence

### Neural decoder

- dataset preparation

- leakage-safe splitting

- SSVEPNet prototype

- training

- validation

- inference

- comparison against classical baseline

### Decision engine

- confidence threshold

- temporal stability

- NONE/abstention

- debounce

- refractory period

### Application

- semantic event interface

- frequency-to-action configuration

- game integration

### Evaluation

- classification accuracy

- confusion matrix

- false triggers

- missed triggers

- detection latency

- end-to-end latency

- repeated-trigger behavior

# 79. Final Development Philosophy

The project should evolve like this:

┌───────────────────────┐

│    PROTOTYPE 1        │

│                       │

│ 1 channel             │

│ 4 frequencies         │

│ classical + neural    │

│ real-time             │

│ simple game           │

└───────────┬───────────┘

│

▼

validate architecture

│

▼

┌───────────────────────┐

│   PROTOTYPE 1.x       │

│                       │

│ optimize latency      │

│ improve robustness    │

│ continuous operation  │

└───────────┬───────────┘

│

▼

┌───────────────────────┐

│    PROTOTYPE 2        │

│                       │

│ 8 channels            │

│ spatial processing    │

└───────────┬───────────┘

│

▼

┌───────────────────────┐

│    PROTOTYPE 3        │

│                       │

│ 16 channels           │

│ advanced spatial      │

│ processing            │

└───────────┬───────────┘

│

▼

┌───────────────────────┐

│    PROTOTYPE 4        │

│                       │

│ N frequencies         │

│ N commands            │

└───────────┬───────────┘

│

▼

┌───────────────────────┐

│   PRODUCTION-LIKE     │

│       BCI             │

│                       │

│ adaptive              │

│ robust                │

│ personalized          │

│ optimized             │

└───────────────────────┘

The guiding principle is:

Build Prototype 1 as a complete system, not as a disposable experiment. But keep Prototype 1 itself deliberately small.

The first implementation should therefore be 1 channel + 4 targets, while the interfaces, tensor shapes, metadata, configuration, decoder contracts, event system, and acquisition boundary are already designed for N channels + N targets.

That gives the team a stable foundation without prematurely solving problems that only become relevant at 8/16 channels or larger target sets.